Microsoft ISA Server 2004

Site-to-Site VPN in
ISA Server 2004

# Introduction

Microsoft® Internet Security and Acceleration (ISA) Server 2004 provides secure site-to-site virtual private network (VPN) functionality.

## Virtual Private Networks

A VPN is the extension of a private network that encompasses links across shared or public networks like the Internet. With a VPN, you can send data between two computers across a shared or public network in a manner that emulates a point-to-point private link. Virtual private networking is the act of creating and configuring a virtual private network.

To emulate a point-to-point link, data is encapsulated, or wrapped, with a header that provides routing information, which allows the data to traverse the shared or public network to reach its endpoint. To emulate a private link, the data is encrypted for confidentiality. Packets that are intercepted on the shared or public network are indecipherable without the encryption keys. The link in which the private data is encapsulated and encrypted is a VPN connection.

VPN connections allow users who work at home or travel to obtain a remote access connection to an organization server using the infrastructure provided by a public network such as the Internet. From the user’s perspective, the VPN is a point-to-point connection between the computer, the VPN client, and an organization server (the VPN server). The exact infrastructure of the shared or public network is irrelevant, because it appears as if the data is sent over a dedicated private link.

VPN connections also allow organizations to have routed connections with other organizations over a public network, such as the Internet, while maintaining secure communications (for example, between offices that are geographically separate). A routed VPN connection across the Internet logically operates as a dedicated wide area network (WAN) link.

By using the ISA Server computer as the VPN server, you can easily manage site-to-site VPN connections, and VPN client access to the corporate network. All VPN connections to the ISA Server computer are logged to the Firewall log, so that you can monitor VPN connections.

ISA Server enables VPN client access using Layer Two Tunneling Protocol (L2TP) over Internet Protocol security (IPSec), which is superior from a security standpoint to the standard Point-to-Point Tunneling Protocol (PPTP) protocol commonly used by VPN servers.

### VPN Connections

There are two types of VPN connections:

Remote access VPN connection

Site-to-site VPN connection

#### Remote access VPN connection

A remote access client makes a remote access VPN connection that connects to a private network. ISA Server provides access to the entire network to which the VPN server is attached. Configuration of remote access VPN connections is discussed in the document VPN Roaming Clients in ISA Server 2004  (http://go.microsoft.com/fwlink?linkid=20745).

#### Site-to-site VPN connection

A router makes a site-to-site VPN connection that connects two portions of a private network. ISA Server provides a connection to the network to which the ISA Server computer is attached. Site-to-site VPN connections are discussed in this document.

### VPN Protocols

There are three VPN protocols for site-to-site connections:

Point-to-Point Tunneling Protocol (PPTP)

Layer Two Tunneling Protocol (L2TP) over Internet Protocol security (IPSec)

Internet Protocol security (IPSec) tunnel mode

#### PPTP

Point-to-Point Tunneling Protocol (PPTP) is a network protocol that enables the secure transfer of data from a remote client to a private enterprise server by creating a virtual private network (VPN) across TCP/IP-based data networks. PPTP supports on-demand, multi-protocol, virtual private networking over public networks such as the Internet. PPTP allows IP traffic to be encrypted, and then encapsulated in an IP header to be sent across a corporate IP network or a public IP network such as the Internet.

#### L2TP over IPSec

Layer Two Tunneling Protocol (L2TP) is an industry-standard Internet tunneling protocol that provides encapsulation for sending Point-to-Point Protocol (PPP) frames across packet-oriented media. L2TP allows IP traffic to be encrypted, and then sent over any medium that supports point-to-point datagram delivery, such as IP. The Microsoft implementation of the L2TP protocol uses Internet Protocol security (IPSec) encryption to protect the data stream from the VPN client to the VPN server. IPSec tunnel mode allows IP packets to be encrypted, and then encapsulated in an IP header to be sent across a corporate IP network or a public IP network such as the Internet.

PPTP connections require only user-level authentication through a PPP-based authentication protocol. L2TP/IPSec connections require the same user-level authentication and, in addition, computer-level authentication using computer certificates.

#### IPSec tunnel mode

Tunneling is the entire process of encapsulation, routing, and decapsulation. Tunneling wraps, or encapsulates, the original packet inside a new packet. This new packet might have new addressing and routing information, which enables it to travel through a network. When tunneling is combined with data confidentiality, the original packet data (as well as the original source and destination) is not revealed to those listening to traffic on the network. After the encapsulated packets reach their destination, the encapsulation is removed, and the original packet header is used to route the packet to its final destination.

The tunnel itself is the logical data path through which the encapsulated packets travel. To the original source and destination peer, the tunnel is usually transparent and appears as just another point-to-point connection in the network path. The peers are unaware of any routers, switches, proxy servers, or other security gateways between the tunnel’s beginning point and the tunnel’s endpoint. When tunneling is combined with data confidentiality, it can be used to provide a virtual private network (VPN).

The encapsulated packets travel through the network inside the tunnel. In this example, the network is the Internet. The gateway might be an edge gateway that stands between the outside Internet and the private network. The edge gateway can be a router, firewall, proxy server, or other security gateway. Also, two gateways can be used inside the private network to protect traffic across untrusted parts of the network.

When Internet Protocol security (IPSec) is used in tunnel mode, IPSec itself provides encapsulation for IP traffic only. The primary reason for using IPSec tunnel mode is interoperability with other routers, gateways, or end-systems that do not support L2TP over IPSec or PPTP VPN tunneling. Interoperability information is provided on the Virtual Private Network Consortium Web site  (http://www.vpnc.org).

# Scenarios

A large corporation often has multiple sites that require communication, for example, a corporate office in New York and a sales office in Washington. The two offices can be connected securely over the Internet using site-to-site virtual private networking. Internet Security and Acceleration (ISA) Server 2004 provides three methods of establishing a site-to-site VPN connection: Internet Protocol security (IPSec) tunnel mode, Layer Two Tunneling Protocol (L2TP) over IPSec, or Point-to-Point Tunneling Protocol (PPTP).

This document considers two scenarios:

- The site you are connecting to is using a third-party product as its VPN server. In this scenario, you should use the IPSec tunnel mode solution.

- The site you are connecting to is using ISA Server 2000, ISA Server 2004, Windows® 2000 Server, or Windows Server™ 2003 as its VPN server. In this scenario, you can use either the L2TP over IPSec solution or the PPTP solution. Of these two, L2TP over IPSec is considered more secure.

# Solutions

Using Internet Security and Acceleration (ISA) Server 2004, configuring a site-to-site VPN connection consists of four general steps.

- Configure the local VPN server, which in this case is an ISA Server computer. This includes the choice of a protocol for the VPN connection.

- Configure ISA Server network rules and access policy. For L2TP over IPSec and for PPTP, you must also configure the general VPN properties for connections initiated by remote VPN sites, because ISA Server views those sites as VPN clients.

- Configure automatic dialing, if your ISA Server computer is connected to the Internet through a dial-up connection.

- Configure the remote VPN server.

You can use one of three protocols to create the VPN connection:

- Internet Protocol security (IPSec) tunnel mode

- Layer Two Tunneling Protocol (L2TP) over IPSec

- Point-to-Point Tunneling Protocol (PPTP)

The following table compares the three protocols.

| Protocol | When to use | Security level | Comments |
| --- | --- | --- | --- |
| IPSec tunnel mode | Connecting to third-party VPN server | High | This is the only option you can use if you are connecting to a non-Microsoft VPN server. |
| L2TP over IPSec | Connecting to an ISA Server 2000 computer, ISA Server 2004 computer, or Windows VPN server | High | Uses Routing and Remote Access. Less complicated than the IPSec tunnel solution, but requires that the remote VPN server be an ISA Server computer or a Windows VPN server. |
| PPTP | Connecting to an ISA Server 2000 computer, ISA Server 2004 computer, or Windows VPN server | Moderate | Uses Routing and Remote Access. Same restrictions as L2TP, but slightly easier to configure. L2TP is considered more secure because it uses IPSec encryption. |

A walk-through for each of these protocols is provided in the sections that follow.

## IPSec Tunnel Solution - Walk-through

Use the IPSec tunnel solution in a scenario where you are using ISA Server 2004 as your VPN server, and the site you are connecting to is using a third-party product or ISA Server 2004 as its VPN server, as shown in the network topology section that follows.

### IPSec Tunnel Solution Network Topology

The figure describes a possible network topology for the IPSec tunnel solution.

As shown in the figure, the main office is using ISA Server 2004 as its VPN server. ISA Server 2004 is on a computer running either Windows 2000 Server or Windows Server 2003.

The branch office may be running one of several third-party VPN servers. Configuration information for third-party VPN servers may be obtained from the Virtual Private Network Consortium Web site  (http://www.vpnc.org). Alternatively, the branch may be using ISA Server 2004, Windows 2000 Server, or Windows Server 2003 as its VPN server.

### IPSec Tunnel Walk-through Procedure 1: Add a Remote Site Network

When you configure a site-to-site VPN in ISA Server, you are establishing a new network: the remote site, recognized by the ISA Server computer as a remote VPN. The following procedure sets up that network.

- Open Microsoft ISA Server Management.

- In the console tree, select Virtual Private Networks (VPN).

- In the details pane, select the Remote Sites tab.

- In the tasks pane, on the Tasks tab, click Add Remote Site Network to start the New Network Wizard.

- On the Welcome page, provide a name for the new network, such as Washington Sales Office IPSec Tunnel, and then click Next. The network name is limited to 200 characters.

- On the VPN Protocol page, select IP Security protocol (IPSec) tunnel mode, and then click Next.

- On the Connection Settings page, you must supply the IP addresses for the remote and local VPN servers. In Remote VPN gateway IP address, provide the IP address that connects the remote VPN server to the Internet.

- Click Networks to open the Allowed Networks dialog box. Select the network that represents the VPN gateway for the ISA Server computer. Typically, and by default, this will be the External network, because the VPN connection will be across the Internet. Click OK to close the Allowed Networks dialog box.

- In Local VPN gateway IP address, select the IP address of the network adapter that connects the ISA Server computer to the network you selected in the previous step.

- On the IPSec Authentication page, select Use pre-shared key for authentication. This is the default IPSec authentication method for the IPSec tunnel solution. Enter the pre-shared key, and then click Next.

- On the Network Addresses page, click Add and add the address ranges of the remote network. You can obtain this information from the administrator of the remote network.

- On the summary page, review the configuration, and then click Finish.

- In the ISA Server details pane, click Apply to apply the changes to ISA Server.

IPSec Tunnel Walk-through Procedure 2: Create Network Rules and Firewall Policy

Note

Advantages and disadvantages of pre-shared keys

Pre-shared key authentication does not require the hardware and configuration investment of a public key infrastructure (PKI), which is necessary for using computer certificates for IPSec authentication. Pre-shared keys are simple to configure on a local VPN server.

A single local VPN server can utilize only one pre-shared key for all L2TP/IPSec connections that require a pre-shared key for authentication. Therefore, you must issue the same pre-shared key to all VPN remote sites that connect to the local VPN server using a pre-shared key, in the L2TP/IPSec scenario. This limitation reduces the security of the deployment and increases the probability of error. Moreover, if the pre-shared key on a local VPN server is changed, a remote VPN server with a manually configured pre-shared key will be unable to connect to that server until the pre-shared key on the remote VPN server is changed.

Unlike certificates, the origin and the history of a pre-shared key cannot be determined. For these reasons, the use of pre-shared keys to authenticate IPSec connections is considered a relatively weak authentication method. If you want a long term, strong authentication method, you should consider using a PKI. This would require installation of digital certificates from the same certification authority on both the local and the remote VPN servers. For more information about digital certificates, see Digital Certificates for ISA Server 2004  (http://go.microsoft.com/fwlink/?LinkId=20794).

After you have created the remote VPN, ISA Server views it as it does any other network connected to the ISA Server computer. You should now create:

- Network rules to establish whether the network has a network address translation (NAT) or route relationship with the other networks connected to the ISA Server computer.

- Access rules to control access to and from the remote network. When you create access rules, you can choose to have requests that match the rules written to the ISA Server log. You can then access the log to review access to and from the remote site network.

For information about network rules and access rules, see ISA Server 2004 Help.

### IPSec Tunnel Walk-through Procedure 3: Configure Automatic Dialing

If your ISA Server computer is connected to the Internet through a dial-up connection, you can configure ISA Server to automatically dial the connection when a client computer sends a request to the remote VPN. The procedure for configuring automatic dialing is provided in Appendix A: Configuring Automatic Dialing in this document.

### IPSec Tunnel Walk-through Procedure 4: Configure the Remote VPN Server

The remote VPN server must be configured to connect to the ISA Server computer in IPSec tunnel mode, according to manufacturer instructions. You may also find useful information on the Virtual Private Network Consortium Web site  (http://www.vpnc.org).

ISA Server provides a summary of the information needed to configure the remote server. To obtain the summary, follow these steps.

- Open Microsoft ISA Server Management.

- In the console tree, select Virtual Private Networks (VPN).

- In the details pane, select the Remote Sites tab.

- Select the network you created in Procedure 1.

- In the tasks pane, on the Tasks tab, click View Remote Site IPSec Policy. The dialog box that appears contains the information that the administrator of the remote VPN server needs so that the connection from the remote VPN server to the ISA Server computer can be configured.

### IPSec Tunnel Walk-through Procedure 5: Test and Monitor the Connection

Test and monitor the VPN connection as described in Appendix B: Testing and Monitoring the VPN Connection in this document.

### IPSec Tunnel Walk-through Procedure 6: Configure Advanced IPSec Settings

You can configure advanced IPSec settings, specifically, the Phase I and Phase II Internet Key Exchange (IKE) protocol settings. Follow these steps to access the settings.

- Open Microsoft ISA Server Management.

- In the console tree, select Virtual Private Networks (VPN).

- In the details pane, select the Remote Sites tab, and double-click the remote site network for which you want to configure the IKE settings, to open its properties.

- On the Connection tab, click IPSec settings to open the IPSec Configuration dialog box. On this dialog box, you can select the Phase I and Phase II tabs, and then modify the settings.

- Click OK to close the IPSec Configuration dialog box, and OK to close the network properties dialog box.

## L2TP over IPSec Solution - Walk-through

Use the L2TP over IPSec solution in a scenario where you are using ISA Server 2004 as your VPN server, and the site you are connecting to is using Windows 2000 Server, Windows Server 2003, ISA Server 2000, or ISA Server 2004 as a VPN server. ISA Server 2004 uses the Windows Routing and Remote Access service to establish the L2TP over IPSec VPN connection.

### L2TP over IPSec Solution Network Topology

The figure describes a possible network topology for the L2TP over IPSec solution.

As shown in the figure, the main office is using an ISA Server 2004 computer as its VPN server. ISA Server 2004 is on a computer running either Windows 2000 Server or Windows Server 2003.

The branch office is running Windows 2000 Server, Windows Server 2003, ISA Server 2000, or ISA Server 2004 as its VPN server.

### L2TP over IPSec Walk-through Procedure 1: Add a Remote Site Network

When you configure a site-to-site VPN in ISA Server, you are establishing a new network: the remote site, recognized by the ISA Server computer as a remote VPN. The following procedure sets up that network.

- Open Microsoft ISA Server Management.

- In the console tree, select Virtual Private Networks (VPN).

- In the details pane, select the Remote Sites tab.

- In the tasks pane, on the Tasks tab, click Add Remote Site Network to start the New Network Wizard.

- On the Welcome page, provide a name for the new network, such as Washington Sales Office L2TP over IPSec, and then click Next.

- On the VPN Protocol page, select Layer Two Tunneling Protocol (L2TP) over IPSec, and then click Next.

- On the Remote Site Gateway page, supply the name or IP address for the remote VPN servers, and then click Next.

- On the Remote Authentication page, you can select to allow outgoing connections from the local site to the remote site. If you enable this option, you must provide a user name, domain, and password for the connection. If you do not enable this option, you will not be able to establish outgoing connections to the remote VPN site, although you will be able to accept connections from that site. Click Next.

- The Local Authentication page provides a reminder that a user with dial-in properties must be configured on the local network for the remote network to be able to initiate a connection to the local network. This user must have the same name as the remote network.

On the Local Authentication page, Click Next.

Note

To configure a user with dial-in properties, open Computer Management, and in Local Users and Groups, select Users. Double-click a user to open its properties, and select the Dial-in tab. Under Remote Access Permission (Dial-in or VPN), select Allow access. Click OK to close the properties dialog box.

- On the L2TP/IPSec Authentication page, you have the option of adding pre-shared key IPSec authentication as a backup authentication method. If you select this option, provide the pre-shared key. Digital certificates are used as the primary authentication method, if you have installed digital (IPSec) certificates from the same trusted certificate authority on both the local and remote VPN servers. For more information about Digital Certificates, see Digital Certificates for ISA Server 2004  (http://go.microsoft.com/fwlink/?LinkId=20794). Click Next.

On the Network Addresses page, click Add and add the address ranges of the remote network. You can obtain this information from the administrator of the remote network.

Note

Advantages and disadvantages of pre-shared keys

Pre-shared key authentication does not require the hardware and configuration investment of a public key infrastructure (PKI), which is necessary for using computer certificates for IPSec authentication. Pre-shared keys are simple to configure on a local VPN server.

A single local VPN server can utilize only one pre-shared key for all L2TP/IPSec connections that require a pre-shared key for authentication. Therefore, you must issue the same pre-shared key to all VPN remote sites that connect to the local VPN server using a pre-shared key, in the L2TP/IPSec scenario. This limitation reduces the security of the deployment and increases the probability of error. Moreover, if the pre-shared key on a local VPN server is changed, a remote VPN server with a manually configured pre-shared key will be unable to connect to that server until the pre-shared key on the remote VPN server is changed.

Unlike certificates, the origin and the history of a pre-shared key cannot be determined. For these reasons, the use of pre-shared keys to authenticate IPSec connections is considered a relatively weak authentication method. If you want a long term, strong authentication method, you should consider using a PKI. This would require installation of digital certificates from the same certificate authority on both the local and the remote VPN servers. For more information about digital certificates, see Digital Certificates for ISA Server 2004  (http://go.microsoft.com/fwlink/?LinkId=20794).

- On the summary page, review the configuration, and then click Finish.

### L2TP over IPSec Walk-through Procedure 2: Set General VPN Properties

ISA Server views site-to-site VPN connections initiated by the remote site as it does a connection from any remote VPN client. To ensure that a secure connection can be established, you must configure the general VPN properties.

In the tasks pane, on the Tasks tab, under General VPN Configuration, select VPN Access Points. This opens the Virtual Private Networks properties dialog box to the Allowed Networks tab. On this tab, select the remote VPN you created, so that the remote network will be able to initiate a connection to the local VPN server.

Note

When you set the general VPN configuration in the Virtual Private Networks properties dialog box, the settings will apply to any VPN connection initiated by a remote client, whether a roaming client or a remote site.

- Select the Address Assignment tab. We recommend that you use a Dynamic Host Configuration Protocol (DHCP) server to dynamically assign IP addresses to VPN clients when they connect. Select Dynamic Host Configuration Protocol. From the drop-down menu, select Use the following network to obtain DHCP, DNS and WINS, select Internal, and then click OK, to indicate that the DHCP server is on the Internal network. Click OK.

@@Alt Text@@

Tip

To use DHCP to assign IP addresses to VPN clients, you must have a DHCP server located on the Internal network side of the ISA Server computer, as shown in the following figure.

Select the Authentication tab, and then select the authentication methods that the incoming connection requests will use. If the connection will use a pre-shared key, also select Allow custom IPSec policy for L2TP connection and provide the pre-shared key.

Or, you must have a router specifically configured to pass DHCP requests to a DHCP server behind the router. If you do not want to set up a DHCP server or configure a router to pass DHCP requests, select Static Address Pool in this step, rather than Dynamic Host Configuration Protocol. Then click Add to add IP address ranges to the static address pool. Note that IP addresses in the static address pool cannot be addresses that are included in the Internal network. You must provide one more IP address in the static address pool than the expected number of remote VPN connections (this includes remote site and roaming client connections). If necessary, edit the Internal network to remove addresses, so that they can be included in the static address pool.

To remove the IP addresses included in the Internal network, in the ISA Server console, expand the Configuration node and click Networks. In the details pane, double-click the Internal network. On the Addresses tab, select a range of IP addresses, and click Remove to remove that range.

If you use a DHCP server to assign IP addresses on the Internal network, but will assign a group of IP addresses from the Internal network to be a static pool for VPN clients, you must configure the DHCP server to not assign those addresses.

- If you are using RADIUS to authenticate the user whose credentials are being used for the remote VPN connection, select the RADIUS tab and configure RADIUS usage.

- Click OK to close the Virtual Private Networks properties dialog box.

- In the ISA Server details pane, click Apply to apply the changes to ISA Server. It may take a few moments for the changes to be applied.

### L2TP over IPSec Walk-through Procedure 3: Create Network Rules and Firewall Policy

After you have created the remote VPN, ISA Server views it as it does any other network connected to the ISA Server computer. You should now create:

- Network rules to establish whether the network has a network address translation (NAT) or route relationship with the other networks connected to the ISA Server computer.

- Access rules to control access to and from the remote network. When you create access rules, you can choose to have requests that match the rules written to the ISA Server log. You can then access the log to review access to and from the remote site network.

For information about network rules and access rules, see ISA Server 2004 Help.

### L2TP over IPSec Walk-through Procedure 4: Configure Automatic Dialing

If your ISA Server computer is connected to the Internet through a dial-up connection, you can configure ISA Server to automatically dial the connection when a client computer sends a request to the remote VPN. The procedure for configuring automatic dialing is provided in Appendix A: Configuring Automatic Dialing in this document.

### L2TP over IPSec Walk-through Procedure 5: Configure the Remote VPN Server

Configure the remote VPN server to connect to the ISA Server computer in L2TP over IPSec mode.

### L2TP over IPSec Walk-through Procedure 6: Test and Monitor the Connection

Test and monitor the VPN connection as described in Appendix B: Testing and Monitoring the VPN Connection in this document.

## PPTP Solution - Walk-through

Use the PPTP solution in a scenario where you are using ISA Server 2004 as your VPN server, and the site you are connecting to is using Windows 2000 Server, Windows Server 2003, ISA Server 2000, or ISA Server 2004 as a VPN server. ISA Server 2004 uses the Windows Routing and Remote Access service to establish the PPTP VPN connection.

PPTP Solution Network Topology

Note

LT2P over IPSec is a more secure solution for this scenario, because communication takes place over HTTPS, and the VPN servers are validated using digital certificates. PPTP is a simpler solution, because it does not require the installation of digital certificates on the local and remote VPN servers.

The figure describes a possible network topology for the PPTP solution.

As shown in the figure, the main office is using ISA Server 2004 as its VPN server. ISA Server 2004 is on a computer running either Windows 2000 Server or Windows Server 2003.

The branch office is running Windows 2000 Server, Windows Server 2003, ISA Server 2000 or ISA Server 2004 as its VPN server.

### PPTP Walk-through Procedure 1: Add a Remote Site Network

When you configure a site-to-site VPN in ISA Server, you are establishing a new network: the remote site, recognized by the ISA Server computer as a remote VPN. The following procedure sets up that network.

- Open Microsoft ISA Server Management.

- In the console tree, select Virtual Private Networks (VPN).

- In the details pane, select the Remote Sites tab.

- In the tasks pane, on the Tasks tab, click Add Remote Site Network to start the New Network Wizard.

- On the Welcome page, provide a name for the new network, such as Washington Sales Office PPTP, and then click Next.

- On the VPN Protocol page, select Point-to-Point Tunneling Protocol (PPTP), and then click Next.

- On the Remote Site Gateway page, supply the name or IP address for the remote VPN servers, and then click Next.

- On the Remote Authentication Page, you can select to allow outgoing connections from the local site to the remote site. If you enable this option, you must provide a user name, domain, and password for the connection. If you do not enable this option, you will not be able to establish outgoing connections to the remote VPN site, although you will be able to accept connections from that site. Click Next.

- The Local Authentication page provides a reminder that a user with dial-in properties must be configured on the remote network. This user must have the same name as the remote network. Click Next.

On the Network Addresses page, click Add and add the address ranges of the remote network. You can obtain this information from the administrator of the remote network.

Note

To configure a user with dial-in properties, open Computer Management, and in Local Users and Groups, select Users. Double-click a user to open its properties, and select the Dial-in tab. Under Remote Access Permission (Dial-in or VPN), select Allow access. Click OK to close the properties dialog box.

- On the summary page, review the configuration, and then click Finish.

- In the ISA Server details pane, click Apply to apply the changes to ISA Server.

### PPTP Walk-through Procedure 2: Set General VPN Properties

ISA Server views site-to-site VPN connections initiated by the remote site as it does a connection from any remote VPN client. To ensure that a secure connection can be established, you must configure the general VPN properties.

In the tasks pane, on the Tasks tab, under General VPN Configuration, select VPN Access Points. This opens the Virtual Private Networks properties dialog box to the Allowed Networks tab. On this tab, select the remote VPN you created, so that the remote network will be able to initiate a connection to the local VPN server.

Note

When you set the general VPN configuration in the Virtual Private Networks properties dialog box, the settings will apply to any VPN connection initiated by a remote client, whether a roaming client or a remote site.

- Select the Address Assignment tab. We recommend that you use a Dynamic Host Configuration Protocol (DHCP) server to dynamically assign IP addresses to VPN clients when they connect. Select Dynamic Host Configuration Protocol. From the drop-down menu, select Use the following network to obtain DHCP, DNS and WINS, select Internal, and then click OK, to indicate that the DHCP server is on the Internal network. Click OK.

Tip

To use DHCP to assign IP addresses to VPN clients, you must have a DHCP server located on the Internal network side of the ISA Server computer, as shown in the following figure.

Select the Authentication tab, and then select the authentication methods that the incoming connection requests will use.

Or, you must have a router specifically configured to pass DHCP requests to a DHCP server behind the router. If you do not want to set up a DHCP server or configure a router to pass DHCP requests, select Static Address Pool in this step, rather than Dynamic Host Configuration Protocol. Then click Add to add IP address ranges to the static address pool. Note that IP addresses in the static address pool cannot be addresses that are included in the Internal network. You must provide one more IP address in the static address pool than the expected number of remote VPN connections (this includes remote site and roaming client connections). If necessary, edit the Internal network to remove addresses, so that they can be included in the static address pool.

To remove the IP addresses included in the Internal network, in the ISA Server console, expand the Configuration node and then click Networks. In the details pane, double-click the Internal network. On the Addresses tab, select a range of IP addresses, and click Remove to remove that range.

If you use a DHCP server to assign IP addresses on the Internal network, but will assign a group of IP addresses from the Internal network to be a static pool for VPN clients, you must configure the DHCP server to not assign those addresses.

- If you are using RADIUS to authenticate the user whose credentials are being used for the remote VPN connection, select the RADIUS tab and configure RADIUS usage.

- Click OK to close the Virtual Private Networks properties dialog box.

- In the ISA Server details pane, click Apply to apply the changes to ISA Server. It may take a few moments for the changes to be applied.

### PPTP Walk-through Procedure 3: Create Network Rules and Firewall Policy

After you have created the remote VPN, ISA Server views it as it does any other network connected to the ISA Server computer. You should now create:

- Network rules to establish whether the network has a network address translation (NAT) or route relationship with the other networks connected to the ISA Server computer.

- Access rules to control access to and from the remote network. When you create access rules, you can choose to have requests that match the rules written to the ISA Server log. You can then access the log to review access to and from the remote site network.

For information about network rules and access rules, see ISA Server 2004 Help.

### PPTP Walk-through Procedure 4: Configure Automatic Dialing

If your ISA Server computer is connected to the Internet through a dial-up connection, you can configure ISA Server to automatically dial the connection when a client computer sends a request to the remote VPN. The procedure for configuring automatic dialing is provided in Appendix A: Configuring Automatic Dialing in this document.

### PPTP Walk-through Procedure 5: Configure the Remote VPN Server

Configure the remote VPN server to connect to the ISA Server computer in PPTP mode.

### PPTP Walk-through Procedure 6: Test and Monitor the Connection

Test and monitor the VPN connection as described in Appendix B: Testing and Monitoring the VPN Connection in this document.

## Appendix A: Configuring Automatic Dialing

To configure automatic dialing:

- Open Microsoft ISA Server Management.

- In the console tree, expand the Configuration node and select Networks.

- In the tasks pane, on the Tasks tab, click Configure Dial-up preferences to open the Dialing Configuration dialog box.

- To allow automatic dialing, select Allow automatic dialing to this network, and from the drop-down menu, select the remote VPN you created. Note that you can only configure automatic dialing to one network.

- If the dial-up connection is your default gateway, select Configure Dial-up connection to be the default gateway.

- In Dial-up connection in the dialog box, in Use the following dial-up connection, provide the name of the dial-up connection, or click Select to select a connection.

If a specific account and password are needed to use the dial-up connection, in Dial-up account in the dialog box, click Set Account and provide the user and password information. Click OK to close the Set Account dialog box.

Note

Create dial-up connections by using the Windows New Connection Wizard.

If you choose a dial-up entry that is a VPN connection, you will experience connectivity problems, because  the PPTP and L2TP/IPSec protocols that are used to establish VPN connections are blocked by default in ISA Server. To avoid this, create access rules that allow PPTP and L2TP/IPSec traffic.

- Click OK to close the Dialing Configuration dialog box.

## Appendix B: Testing and Monitoring the VPN Connection

Follow these steps to test and monitor the VPN connection.

##### Step 1: Test the connection

After you have created the connection, test it by trying to access a computer on the remote network from a computer on the local network (for which network rules and access policy allow access). If you can access the computer on the remote network, you have correctly configured the site-so-site VPN connection.

##### Step 2: Check ISA Server for connection information

This procedure takes place on the ISA Server computer.

- In the ISA Server console tree, click Monitoring.

- On the Sessions tab, verify whether your VPN session is listed.

The example companies, organizations, products, domain names, e-mail addresses, logos, people, places, and events depicted herein are fictitious. No association with any real company, organization, product, domain name, e-mail address, logo, person, places, or events is intended or should be inferred.

Information in this document, including URL and other Internet Web site references, is subject to change without notice. Unless otherwise noted, the example companies, organizations, products, people, and events depicted herein are fictitious and no association with any real company, organization, product, person, or event is intended or should be inferred. Complying with all applicable copyright laws is the responsibility of the user. Without limiting the rights under copyright, no part of this document may be reproduced, stored in or introduced into a retrieval system, or transmitted in any form or by any means (electronic, mechanical, photocopying, recording, or otherwise), or for any purpose, without the express written permission of Microsoft Corporation.

Microsoft may have patents, patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. Except as expressly provided in any written license agreement from Microsoft, the furnishing of this document does not give you any license to these patents, trademarks, copyrights, or other intellectual property.

©2003 Microsoft Corporation. All rights reserved.

Microsoft, Active Directory, Outlook, Windows, Windows Media, and Windows NT are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries/regions.

Do you have comments about this document? Send feedback.