(this  image is similar with the real exam )

- Radius server : 172.16.149.1

- Key : rad123

- Server:  Vlan 40 is a new vlan which is used to access server

- User: vlan 20, connect to ASW1

- Ip users: 172.16.40.0/24

Config

!!!! Console to ASW1 from PC console 1

ASW1(config)#aaa new-model

ASW1(config)#radius-server host 172.16.149.1 key rad123

ASW1(config)#aaa authentication dot1x default group radius

ASW1(config)#dot1x system-auth-control

ASW1(config)#inter fastEthernet 0/1

ASW1(config-if)#dot1x port-control auto

ASW1(config-if)#exit

ASW1#copy run start

!!! Console to DSW1 from PC console 2

DSW1(config)#ip access-list standard 10

DSW1(config-ext-nacl)#permit ip 172.16.40.0 0.0.0.255

DSW1(config-ext-nacl)#exit

DSW1(config)#vlan access-map PASS 10

DSW1(config-access-map)#match ip address 10

DSW1(config-access-map)#action forward

DSW1(config-access-map)#exit

DSW1(config)#vlan access-map PASS 20

DSW1(config-access-map)#action drop

DSW1(config-access-map)#exit

DSW1(config)#vlan filter PASS vlan-list 40

DSW1#copy run start