Mình đang thực hiện bài Lab VPN theo mô hình như trên, đã cấu hình VPN thành công

- Máy 192.168.100.1 và 192.168.150.1 có thể truy cập lẫn nhau.

- Nhưng máy 192.168.150.1 ở LAN2  không thể truy cập vào máy 192.168.200.1 ở vùng DMZ ở LAN1

Nhờ mọi người giải quyết vấn đề này giúp với. (cấu nhình cho phép máy 192.168.150.1 có thể access đến máy 192.168.100.1 và 192.168.200.1)

Cấu hình PIX1

: Saved

: Written by enable_15 at 01:47:53.136 UTC Tue Sep 30 2008

!

PIX Version 8.0(2)

!

hostname pixfirewall

enable password 8Ry2YjIyt7RRXU24 encrypted

names

!

interface Ethernet0

nameif outside

security-level 0

ip address 203.162.2.2 255.255.255.0

!

interface Ethernet1

nameif inside

security-level 100

ip address 192.168.100.254 255.255.255.0

!

interface Ethernet2

nameif dmz

security-level 50

ip address 192.168.200.254 255.255.255.0

!

interface Ethernet3

shutdown

no nameif

no security-level

no ip address

!

interface Ethernet4

shutdown

no nameif

no security-level

no ip address

!

passwd 2KFQnbNIdI.2KYOU encrypted

ftp mode passive

access-list vpnsite2site extended permit ip 192.168.0.0 255.255.0.0 192.168.0.0 255.255.0.0

pager lines 24

mtu outside 1500

mtu inside 1500

mtu dmz 1500

no failover

icmp unreachable rate-limit 1 burst-size 1

asdm image flash:/asdm-603.bin

no asdm history enable

arp timeout 14400

nat-control

nat (inside) 0 access-list vpnsite2site

route outside 0.0.0.0 0.0.0.0 203.162.2.1 1

timeout xlate 3:00:00

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout uauth 0:05:00 absolute

dynamic-access-policy-record DfltAccessPolicy

no snmp-server location

no snmp-server contact

snmp-server enable traps snmp authentication linkup linkdown coldstart

crypto ipsec transform-set MYSET esp-des esp-none

crypto map abcmap 1 match address vpnsite2site

crypto map abcmap 1 set peer 203.162.3.2

crypto map abcmap 1 set transform-set MYSET

crypto map abcmap interface outside

crypto isakmp identity address

crypto isakmp enable outside

crypto isakmp policy 1

authentication pre-share

encryption des

hash sha

group 2

lifetime 86400

no crypto isakmp nat-traversal

telnet timeout 5

ssh timeout 5

console timeout 0

threat-detection basic-threat

threat-detection statistics access-list

!

class-map inspection_default

match default-inspection-traffic

!

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect h323 h225

inspect h323 ras

inspect netbios

inspect rsh

inspect rtsp

inspect skinny

inspect esmtp

inspect sqlnet

inspect sunrpc

inspect tftp

inspect sip

inspect xdmcp

!

service-policy global_policy global

tunnel-group 203.162.3.2 type ipsec-l2l

tunnel-group 203.162.3.2 ipsec-attributes

pre-shared-key cisco123

prompt hostname context

Cryptochecksum:80e9920c78e9f407ee772a0fa1872e12

: end

Cấu hình PIX2

: Saved

: Written by enable_15 at 01:51:51.408 UTC Tue Sep 30 2008

!

PIX Version 8.0(2)

!

hostname pixfirewall

enable password 8Ry2YjIyt7RRXU24 encrypted

names

!

interface Ethernet0

nameif outside

security-level 0

ip address 203.162.3.2 255.255.255.0

!

interface Ethernet1

nameif inside

security-level 100

ip address 192.168.150.254 255.255.255.0

!

interface Ethernet2

shutdown

no nameif

no security-level

no ip address

!

interface Ethernet3

shutdown

no nameif

no security-level

no ip address

!

interface Ethernet4

shutdown

no nameif

no security-level

no ip address

!

passwd 2KFQnbNIdI.2KYOU encrypted

ftp mode passive

access-list vpnsite2site extended permit ip 192.168.150.0 255.255.255.0 192.168.0.0 255.255.0.0

pager lines 24

mtu outside 1500

mtu inside 1500

no failover

icmp unreachable rate-limit 1 burst-size 1

asdm image flash:/asdm-603.bin

no asdm history enable

arp timeout 14400

nat-control

nat (inside) 0 access-list vpnsite2site

route outside 0.0.0.0 0.0.0.0 203.162.3.1 1

timeout xlate 3:00:00

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout uauth 0:05:00 absolute

dynamic-access-policy-record DfltAccessPolicy

http server enable

http 0.0.0.0 0.0.0.0 inside

no snmp-server location

no snmp-server contact

snmp-server enable traps snmp authentication linkup linkdown coldstart

crypto ipsec transform-set MYSET esp-des esp-none

crypto map abcmap 1 match address vpnsite2site

crypto map abcmap 1 set peer 203.162.2.2

crypto map abcmap 1 set transform-set MYSET

crypto map abcmap interface outside

crypto isakmp identity address

crypto isakmp enable outside

crypto isakmp policy 1

authentication pre-share

encryption des

hash sha

group 2

lifetime 86400

crypto isakmp policy 65535

authentication pre-share

encryption 3des

hash sha

group 2

lifetime 86400

no crypto isakmp nat-traversal

telnet timeout 5

ssh timeout 5

console timeout 0

threat-detection basic-threat

threat-detection statistics access-list

!

class-map inspection_default

match default-inspection-traffic

!

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum 512

policy-map global_policy

class inspection_default

inspect dns preset_dns_map

inspect ftp

inspect h323 h225

inspect h323 ras

inspect netbios

inspect rsh

inspect rtsp

inspect skinny

inspect esmtp

inspect sqlnet

inspect sunrpc

inspect tftp

inspect sip

inspect xdmcp

!

service-policy global_policy global

tunnel-group 203.162.2.2 type ipsec-l2l

tunnel-group 203.162.2.2 ipsec-attributes

pre-shared-key cisco123

prompt hostname context

Cryptochecksum:7067fdaf34375e85ac98bdb8fde295cf

: end