OPEN LAB

Semester 1

Network diagram, major IP address = 155.55.0.0/16

Given the network as in the diagram above, you are to perform the task:

Section1: Using VLSM to create the IP planning

- LAN1: 20 host addresses

- LAN2: 200 host addresses

- LAN3: 200 host addresses

- LAN4: 100 host addresses

- LAN5: 100 host addresses

- LAN6: 10 host addresses

- WAN Link: 2 host addresses

1.1: IP subnetting table

| Name required addresses | Subnet addresses | Address range | Broadcast address | Network/Prefix |
| --- | --- | --- | --- | --- |
| LAN1 |  |  |  |  |
| LAN2 |  |  |  |  |
| LAN3 |  |  |  |  |
| LAN4 |  |  |  |  |
| LAN5 |  |  |  |  |
| LAN6 |  |  |  |  |
| WAN link |  |  |  |  |

1.2: IP Address assignments

|  | IP Address | Default gateway | Network number |
| --- | --- | --- | --- |
| Router1, S0/1 |  |  |  |
| Router1, F0/0 |  |  |  |
| Router1, Lookback2 |  |  |  |
| Router1, Lookback3 |  |  |  |
| Router2, S0/0 |  |  |  |
| Router2, F0/0 |  |  |  |
| Router2, Lookback4 |  |  |  |
| Router2, Lookback5 |  |  |  |
| DNS Server1 |  |  |  |
| DHCP Server2 |  |  |  |
| PC |  |  |  |
| HTTP Server3 |  |  |  |

Section 2: Using Packet Tracer to perform the tasks:

- Create a physical network (Router 2621XM, Switch 2950-24)

- Basic router/switch configurations:

  - Hostname: Assign the Router/Switches/Server/PC names as in the diagram (e.g. ‘router1’, ‘router2’, … , ‘switch1’, ‘switch2’, … , ‘DNS server1’, … ‘PC client’)

  - Passwords & Banner Message: (Router only)

    - Console and VTY lines are password protected, password=  “netpro”.

    - Set the privileged mod password (enable secret password) to “cisco”.

    - Banner contains the name of the student who configures the device.

Banner (motd) example: Hello, this is router 1, configured by Bui Nhu Lac.

  - Router interfaces:

    - Assign IP addresses and subnet masks.

    - User the clock rate 6400 for DCE port

    - Create descriptions. Interface description example: Connect to Router2.

  - Static default router:

    - Router1: (config)# ip router 0.0.0.0 0.0.0.0 s0/1

    - Router2: (config)# ip router 0.0.0.0 0.0.0.0 s0/0

- Configuring the Servers:

    - Assign an IP address and Subnet mask, Default gateway.

    - Active suitable services

- Configuring the PC client: Obtain an IP address and DNS server address automatically.

= = = = = = = = = = = = = = = = =

Bài tập này đã gồm file cấu hình đính kèm.

Student: Tô Thành Công

Instructor: Phạm Bảo Trung

Mọi ý kiến đóng góp mời các thảo luận tại diễn đàn: http://saobacdau-acad.vn

Thanks and Best Regards