Subnet Calculation

Host IP address: 172.168.130.179

Subnet mask: 255.255.192.0

Fill in the following blanks based on the above information.

Major network information: (Dùng IP với subnet mask default để tính)

1. Network ID Class: ___B____ (A, B, or C)

2. Major network address: 172.168.0.0

3. Major broadcast address:172.168.255.255

4. First host of the major network:172.168. 0.1

5. Last host of the major network: 172.168.255.254

Subnet information: (Dùng IP với subnet mask đề bài đã cho để tính)

6. Number of subnet bits:__18_

7. Number of host bits:_14__

8. Subnet address for the given host IP address: 172.168.128.0

9. Range of host addresses for this subnet:172.168.128.1 --->172.168.191.254

10. Broadcast address for this subnet:_172.168.191.255

Other subnet information:

11. Total number of useable subnets:_____4_____

12. Number of valid host IP addresses per subnet (minus 2, excluding the network and

broadcast addresses):_____214 – 2=16382______

13. Convert given subnet mask to binary:11111111.11111111.11000000.00000000

14. Convert given host IP address to binary:10101100.10101000.10000010.10110011

15. Subnetwork address with slash(/) mask for the assigned subnet:172.168.128.0/18

Given the major network – Subnetting the subnet using VLSM fill the table:

16. 1 network with 4000 hosts, 1 network with 2000 hosts, 2 networks with 1024 hosts and 3 WAN links.

| No. | Subnet address | Address range | Broadcast Address | Network/prefix |
| --- | --- | --- | --- | --- |
| 1 | 172.168.0.0 | 172.168.0.1 => 172.168.15.254 | 172.168.15.255 | 172.168.0.0/20 |
| 1 | 172.168.16.0 | 172.168.16.1 => 172.168.23.254 | 172.168.23.255 | 172.168.16.0/21 |
| 2.1 | 172.168.24.0 | 172.168.24.1=> 172.168.31.254 | 172.168.31.255 | 172.168.24.0/21 |
| 2.2 | 172.168.32.0 | 172.168.32.1 => 172.168.39.254 | 172.168.32.255 | 172.168.32.0/21 |
| 3.1 | 172.168.40.0 | 172.168.401=> 172.168. 40.2 | 172.168. 40.3 | 172.168.40.0/30 |
| 3.2 | 172.168.40.4 | 172.168.40.5=> 172.168.40.6 | 172.168.40.7 | 172.168.40.0/30 |
| 3.3 | 172.168.40.8 | 172.168.40.9=> 172.168.40.10 | 172.168.40.11 | 172.168.40.8/30 |